SOI : Cartographer

Autonomous Multi-Robot Mapping Challenge

Participant Technical Guide

ROS 2 Jazzy  ·  Gazebo Harmonic  ·  Ubuntu 24.04 LTS

# 1. Problem Statement

The objective of this challenge is to design a software system that enables two robots to autonomously explore an unknown environment, build maps using onboard sensors, communicate with each other, and merge their independently generated maps into a single consistent global map.

Robots will begin without prior knowledge of the environment and without knowledge of their relative starting positions. Teams must implement algorithms for mapping, exploration, inter-robot communication, and map merging using the provided simulation framework.

# 2. Provided Resources and Required Setup

## 2.1 Starter Repository Contents

Teams will receive a starter repository containing the following components:

- Pre-built differential drive robot models with LiDAR sensors and odometry

- Gazebo simulation launch infrastructure for spawning two robots

- Training simulation environments for development and testing

- Interface documentation describing available ROS topics and robot frames

## 2.2 Required Software Environment

The following software stack is required for development and testing:

- Ubuntu 24.04 LTS

- ROS 2 Jazzy

- Gazebo Harmonic simulator

- Python or C++ for ROS node development

# 3. Tasks to be Implemented by Teams

The following capabilities must be fully implemented within the team solution package:

- SLAM and local mapping — Each robot must construct a local occupancy grid map using LiDAR data.

- Autonomous navigation — Robots must move through the environment safely while avoiding obstacles.

- Autonomous exploration — Robots must determine exploration targets without human intervention.

- Inter-robot communication — Robots must exchange mapping or exploration information.

- Map alignment and merging — Robots must estimate the relative transform between their maps and merge them into a single global map.

- Exploration coordination — Robots should minimise redundant exploration and distribute tasks efficiently.

# 4. Competition Rules

- Robots must rely only on onboard sensors and odometry for perception.

- Access to simulator ground truth data is strictly prohibited.

- Forbidden topics include /world/*, /gazebo/*, /model_states, and any simulator internal state APIs.

- Robots must operate fully autonomously once launched. Manual teleoperation is not permitted.

- Training worlds provided to teams will not be used during evaluation.

- Final evaluation will use hidden environments not released to participants.

- Robot starting positions will be randomised during evaluation.

- Teams must submit only their solution package. The simulation infrastructure will be controlled by the organisers.

- Organisers will run all submissions in the evaluation environment.

# 5. Repository Structure

The repository is structured as a standard ROS 2 workspace. It contains two top-level packages: the organiser-controlled challenge_bridge package and the participants directory where teams do all their work.

ℹ Participants must only make changes inside src/participants/team_solution_template/. All other directories are controlled by the organisers and will be overwritten at evaluation time.

## 5.1 Top-Level Layout

cartographer/          # Git Repository Root / ROS 2 Workspace

├── .gitignore                 # Ignores build/, install/, log/, __pycache__

├── README.md                  # General rules, dependencies, and setup guide

├── build/                     # [Local Only] Generated by colcon build

├── install/                   # [Local Only] Generated by colcon build

├── log/                       # [Local Only] Generated by colcon build

└── src/                       # All ROS 2 packages live here

├── challenge_bridge/      # ORGANISER PACKAGE

└── participants/          # PARTICIPANT DIRECTORY

The build/, install/, and log/ directories are generated locally by colcon and are excluded from version control via .gitignore. They should never be committed to the repository.

## 5.2 challenge_bridge — Organiser Package

This package is provided and maintained exclusively by the organisers. It contains the full simulation infrastructure: the Gazebo world, robot models, ROS-Gazebo topic bridges, and TF publishers. Teams must not modify any file in this package.

src/challenge_bridge/

├── CMakeLists.txt             # Installs launch/, models/, worlds/ to share/

├── package.xml                # Depends on ros_gz_bridge, tf2_ros, nav_msgs, …

├── launch/

│   └── competition.launch.py  # Launches Gazebo world, spawns both robots,

│                              #   bridges TFs and laser scans into ROS 2

├── models/

│   └── robot.sdf              # Robot URDF/SDF: DiffDrive plugin + LiDAR plugin

└── worlds/

└── training_world_1.sdf   # Sample arena/maze used during development

| Path | Type / Owner | Description |
| --- | --- | --- |
| competition.launch.py | Organiser — Read Only | Entry point that brings up the full simulation: Gazebo, robot spawning, bridge nodes, and TF publishers. |
| robot.sdf | Organiser — Read Only | Defines the physical robot, differential drive controller, and LiDAR sensor with ROS plugin bindings. |
| training_world_1.sdf | Organiser — Read Only | A sample arena for local development. The evaluation environment will differ. |

## 5.3 participants/ — Participant Directory

The participants/ directory contains a README with the full API reference (available topics, message types, QoS settings) and the team_solution_template package which teams clone as their starting point.

src/participants/

├── README.md                  # Technical API Guide: Topics, Message Types, QoS

└── team_solution_template/    # ← YOUR WORKING DIRECTORY

├── package.xml            # Solution dependencies (rclpy, sensor_msgs, nav_msgs)

├── setup.py               # Python packaging setup configuration

├── setup.cfg              # Tells ROS where script entry points are installed

├── team_solution_template/  # Core Python source directory

│   ├── __init__.py

│   └── solution_node.py   # Starter template: Best Effort LaserScan subscriber

└── launch/

└── solution.launch.py # Launch file the evaluation system runs

| Path | Type / Owner | Description |
| --- | --- | --- |
| participants/README.md | Organiser — Read Only | Complete API reference. Lists every subscribable and publishable topic, message type, and required QoS profile. |
| package.xml | Participant — Edit | Declare all ROS and system dependencies your solution requires. Add entries here for any additional packages. |
| setup.py / setup.cfg | Participant — Edit | Python packaging config. Add new entry points here if you split logic into multiple executable nodes. |
| solution_node.py | Participant — Edit | Starter scaffold. Implement your SLAM, navigation, exploration, communication, and map-merging logic here. |
| solution.launch.py | Participant — Edit | The launch file the evaluation system invokes. Ensure this file correctly starts all of your nodes. |

⚠ Note: The evaluation system launches solution.launch.py directly. If your nodes are not started by this file, they will not run during scoring.

# 6. Building and Running the System

The system is composed of two independently launched components: the challenge_bridge (simulation infrastructure) and your team solution. They must be started in the correct order.

## 6.1 Build the Workspace

From the repository root, build all packages with colcon:

cd ~/cartographer

colcon build --symlink-install

source install/setup.bash

The --symlink-install flag is recommended during development: Python source files are symlinked rather than copied, so edits to .py files take effect immediately without rebuilding.

⚠ Note: Run source install/setup.bash in every new terminal before using ros2 commands, or add it to your ~/.bashrc.

## 6.2 Step 1 — Launch the Challenge Bridge

Open a dedicated terminal and launch the simulation environment first. This starts Gazebo, spawns both robots, and brings up all bridge nodes:

# Terminal 1

source ~/cartographer/install/setup.bash

ros2 launch challenge_bridge competition.launch.py

Wait until you see Gazebo open and the terminal output confirms that both robots have been spawned and the bridge topics are active before proceeding to the next step. You should see output similar to:

[INFO] [launch]: All log files can be found below ...

[INFO] [gzserver]: process started with pid ...

[INFO] [bridge_node]: /robot1/scan topic bridged

[INFO] [bridge_node]: /robot2/scan topic bridged

## 6.3 Step 2 — Launch Your Solution

Once the challenge bridge is running, open a second terminal and launch your solution package:

# Terminal 2

source ~/cartographer/install/setup.bash

ros2 launch team_solution_template solution.launch.py

Your solution nodes will connect to the topics published by the bridge. Both robots should begin autonomous operation immediately.

ℹ During evaluation, the organisers will run competition.launch.py first and then solution.launch.py. Ensure your solution starts cleanly from a fresh terminal with no manual pre-configuration steps.

## 6.4 Useful ROS 2 Commands

The following commands are helpful for debugging during development:

# List all active topics

ros2 topic list

# Inspect a topic's message type and publisher info

ros2 topic info /robot1/scan

# Echo messages on a topic (Ctrl+C to stop)

ros2 topic echo /robot1/scan

# Check that your nodes are running

ros2 node list

# View the full TF tree

ros2 run tf2_tools view_frames

# Print TF transform between two frames

ros2 run tf2_ros tf2_echo robot1/odom robot1/base_link

## 6.5 Rebuilding After Changes

If you add new Python files or modify package.xml or setup.py, rebuild before relaunching:

cd ~/cartographer

colcon build --symlink-install --packages-select team_solution_template

source install/setup.bash

Using --packages-select speeds up the build by only rebuilding your package, rather than the entire workspace.

# 7. Final Submission Requirements

Each team must submit the following as a single ROS 2 package:

- A ROS 2 package containing the complete implementation of the team solution

- Launch files required to start the system

- A README describing installation steps and execution instructions

- A 3 to 5 minute demonstration video showing exploration and final map generation

- A technical report of 4 to 6 pages describing system architecture, algorithms, and experimental results

# 8. Evaluation Criteria

Submissions will be scored across five dimensions:

| Criterion | Weight | What is Assessed |
| --- | --- | --- |
| Exploration Coverage | 30% | Percentage of the environment successfully mapped |
| Exploration Efficiency | 20% | Time required to reach high map coverage |
| Map Quality | 20% | Accuracy and consistency of the final merged map |
| Multi-Robot Coordination | 20% | Effectiveness of cooperation between robots |
| Technical Implementation | 10% | Software design quality, modularity, and documentation |